SOFTWARE GUIDE

Installation Setup:
Arduino IDE Configuration

Step 1: Install ESP32 Board Support

1. Install ESP32 board support via Boards Manager: Search for "esp32" by Espressif Systems and install version 2.x or later

Step 2: Install Required Libraries

Open the Library Manager and install the following libraries:

- WebSockets by Markus Sattler (version 2.3.x or later)

- TinyGPSPlusby Mikal Hart (version 1.x)

- ArduinoJson by Benoit Blanchon (version 6.x)

Step 3: Configure Board Settings

- Set your board target to: Tools > Board > esp32 > ESP32 Dev Module

- Set Upload Speed to 115200 and Flash Frequency to 80MHz

Required Library Headers

#include <WiFi.h>

#include <WebSocketsClient.h>

#include <ArduinoJson.h>

#include <TinyGPSPlus.h>

#include <HardwareSerial.h>

Code Architecture

Configuration Block:

const char* ssid         = "YourHotspotName";      // Phone hotspot name

const char* password     = "YourPassword";         // Phone hotspot password

const char* ws_host      = "esp32-tracker.kiyokodyele.com"; // WebSocket server hostname

const uint16_t ws_port   = 443;                    // WSS port (SSL)

const char* device_id    = "ESP32_TeamName";       // "ESP32_" + team name (no spaces)

const char* device_name  = "TeamName";             // Team name

const char* avatar_url   = "https://...";          // Hosted image URL for your marker

GPS and WebSocket Initialization:

void setup() {

// 1. Serial Monitor

Serial.begin(115200);

// 2. GPS on UART2 (pins 16 RX, 17 TX)

GPS_Serial.begin(9600, SERIAL_8N1, 16, 17);

// 3. WiFi Connection

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

// 4. WebSocket with SSL

webSocket.beginSSL(ws_host, ws_port, "/");

webSocket.onEvent(webSocketEvent);

webSocket.setReconnectInterval(5000);

}

Main Loop Structure:

void loop() {

webSocket.loop();

while (GPS_Serial.available() > 0) {

gps.encode(GPS_Serial.read());

}

if (isConnected && (millis() - lastSendTime >= SEND_INTERVAL)) {

lastSendTime = millis();

if (gps.location.isValid()) {

sendLocation(gps.location.lat(), gps.location.lng());

}

}

}

WebSocket Event Handler:

void webSocketEvent(WStype_t type, uint8_t* payload, size_t length) {

switch (type) {

case WStype_CONNECTED:

Serial.println("WebSocket Connected!");

isConnected = true;

registerDevice();

break;

case WStype_DISCONNECTED:

Serial.println("WebSocket Disconnected. Retrying...");

isConnected = false;

break;

case WStype_TEXT:

Serial.print("Message from server: ");

Serial.println((char*)payload);

break;

case WStype_ERROR:

Serial.println("WebSocket Error!");

break;

}

}

Message Protocol (JSON):

Registration Message:

{

"type": "register",

"deviceId": "ESP32_MALDITONG_AY",

"name": "MALDITONG AY",

"avatarUrl": "https://...",

"latitude": 0.0,

"longitude": 0.0

}

Location Update Message:

{

"type": "location",

"latitude": 13.624356,

"longitude": 123.185432,

"avatarUrl": "https://..."

}

GPS Fix and Outdoor Operation:

Understanding GPS Fix Time

Cold Start GPS Acquisition:

A cold start (first power-on outdoors) can take 30 seconds to several minutes to acquire a satellite fix. During this period, gps.location.isValid() returns false.

You can monitor progress in the Serial Monitor:

gps.satellites.value() — Should increase as more satellites are acquired (need at least 4 for a position fix)

gps.hdop.value() — Horizontal Dilution of Precision; lower is better (under 200 is acceptable)

Testing Sequence:

- Flash your sketch and open the Serial Monitor at 115200 baud

- Take the assembled device outdoors or near a window with clear sky view

- Wait for "WiFi connected!" confirmation, followed by "WebSocket Connected!" confirmation

- Monitor the GPS output — Wait for "Location: VALID" before beginning to walk and test

- Verify your team marker appears on the live campus map shown by the instructor (https://esp32-tracker.kiyokodyele.com/) ← Visit to confirm your code works