TROUBLESHOOTING GUIDE

| Symptom | Likely Cause and Fix |
| --- | --- |
| WiFi keeps reconnecting in a loop | SSID or password is wrong. Mobile Hotspot is not broadcasting or is on 5GHz band only — switch to 2.4GHz. |
| WebSocket never connects | Incorrect ws_host or port. Confirm the server address with the instructor. Ensure SSL is enabled (beginSSL, not begin). |
| "Waiting for GPS fix" never resolves | Move outdoors and wait. GPS cold start can take up to 3 minutes. |
| Serial Monitor shows garbled text | Baud rate mismatch. Ensure Serial Monitor is set to 115200. |
| Device shows on map at wrong location | Stale GPS data from a previous session. Power cycle and wait for a fresh fix. |
| JSON parse errors in server logs | Malformed JSON from ArduinoJson — ensure StaticJsonDocument size (256 bytes) is sufficient and you call serializeJson() correctly. |